Bellman Conservation Law-based Large-Scale Pedestrian Flow

Control via Passage Direction and Entrance-Rate Optimization in

Metropolitan Tourism Hotspots

Bingyu Wei, Cuiling Li, Rongyong Zhao, Zhengyang Shi, Yuxin Cai, and Lingchen Han

Abstract—Open   scenic   areas   and   large   event   venues   can develop dense pedestrian flows that are hard to evaluate before an   event.  Existing   practice   still   relies  heavily   on   empirical rules  and  post-event  review,  which  leaves  limited  support  for comparing  control  plans  before  deployment.  In  these  settings, managers  mainly  choose  passage  directions,  such  as  one-way, bidirectional, or closed, and adjust time-varying entrance release rates. Existing models rarely optimize these two controls together. This paper develops a macroscopic crowd-control model based on the Hughes continuum framework. Local admissible direction sets represent one-way, bidirectional, and closed passages, while anisotropic  mobility  tensors  encode  channel-guidance  effects. Internal  channel  entrance-rate  controls  are  added  so  that  one policy specifies both passage directions and release rates. We then solve the mixed discrete-continuous problem with a Hierarchical Constrained Mixed-variable Black-box Optimization (HCMBO) algorithm.  In  a  simplified  scenic-platform  scenario,  the  model reproduces multi-stage movement, channel guidance, and direc- tion constraints. Under a weighted objective combining efficiency, safety exposure, load balance, waiting, and smoothness, HCMBO gives the lowest mean score within the tested budget. Its mean objective is  about 3.9%  lower  than  TPE-Mixed  BO  and  lower than the other tested baselines. These results give a simulation- based basis for comparing routing and entrance-release strategies in open public spaces.

Index Terms—Crowd dynamics modeling, crowd flow control, mixed-variable black-box optimization.

I.  INTRODUCTION

L

ARGE-SCALE  crowd  gatherings  can  threaten  public safety when local congestion grows into a stampede risk [1], [2]. Holiday tourism, public events, and sports gatherings place short, intense pedestrian loads on core urban areas [3], [4], [5]. This motivates crowd-control models that can compare routing and entrance-release strategies before an event, rather than relying only on field experience  and post-event review [6], [7].

Crowd motion at large events depends on individual speeds, destinations,  route  preferences,  visible  paths,  obstacles,  and density feedback [4], [8], [9]. At the aggregate level, these fac- tors appear as destination-oriented flow, passage convergence, bottleneck queuing, and responses to direction rules [10], [11]. In  open  scenic  areas,  visitors  often  move  through  stages: entering  the  site,  touring  along  a  main  corridor,  choosing lateral exit passages, and returning through lower streets [12]. Static capacity analysis and single shortest-path models do not

Bingyu Wei, Cuiling Li, Rongyong Zhao, Yuxin Cai, and Lingchen Han are with Tongji University, Shanghai, China.

Zhengyang Shi is with Huangpu Public Security Bureau, Shanghai, China.

capture these  stage changes or the congestion  shared across routes.

This study focuses on open scenic areas, waterfront sight- seeing  platforms,  and  historic  commercial  districts,  where pedestrian routes often remain open and partially reversible. Unlike enclosed venues such as stadiums and subway stations with  well-defined  boundaries  [13],  [14],  such  areas  consist of open  spaces, main tourist corridors, multiple lateral con- necting  passages,  and  return  flow  lines.  In  these   settings, visitors  typically  undergo  a  continuous  behavioral  chain  of “entry–touring along the main corridor–choosing passages to leave–returning” [15]. Their movement is therefore governed jointly by spatial geometry, control regulations, and historical behavioral preferences [16]. Taking the Shanghai Nanjing East Road–Bund area as a concrete example, visitors approach the Bund from Nanjing East Road, ascend to the Bund sightseeing platform via multiple stepped passages, tour along the water- front, then exit through several southern passages and return along  lower  streets.  This  scenario  can  be  abstracted  as  the typical  “Bund   sightseeing  platform–multi-stepped  passage” crowd flow scenario. Three operational issues follow from this layout. Direction settings on one passage can reorganize flow elsewhere.  Behavioral  transitions  between  the  main  tourist corridor  and lateral passages make  single  origin–destination models insufficient. Local congestion and global route choice also  feed back into  each  other,  so  a  control  change  in  one passage may redistribute loads in nearby areas.

Existing  large-scale  event  security  management  practices typically  integrate  system  dynamics,  system  coupling,  and system safety theory, combined with high-risk zone delineation and historical crowd behavior patterns, to formulate a series of control measures [19], [20]. These measures include deploying security  personnel  at  critical  locations,  erecting  temporary barriers  for  isolation  and  diversion,  implementing  one-way organization on key passages, and applying batch release in core areas [21], [22]. For instance, during major holidays, the Shanghai Bund  area  adopts batch release, passage  direction regulation,  south-entry  north-exit  routing,  and  reduction  of head-on conflicts  [23];  similarly, Nanluoguxiang main  street in Beijing implements one-way passage measures (south entry, north  exit)  during  peak  periods   [24],  consistent  in  control mechanism with the Shanghai Bund approach.

Current practice in high-pedestrian-flow urban areas still de- pends on manual experience and static contingency plans [25]. Many schemes do not quantify how passage direction, entrance capacity, and phase-dependent route preference change density

(a)                                                                                                  (b)

# Fig. 1.  Field-control context of the Shanghai Nanjing East Road–Bund area. (a) Large-scale holiday crowd under one-way passage organization; image source:

[17]. (b) Entrance-capacity control at a main visitor inflow location; image source: [18].

evolution. Single-point statistics and local density monitoring make  it hard to  follow the  coupling  among  direction rules, entrance metering, local optimal directions, density evolution, and exit-load distribution [26]. In open scenic areas, ignoring route preference  and phase  transition  can  misjudge passage load and high-risk locations. Simulation-optimization studies often treat the crowd model as a black-box evaluator, so the search does not use the  structure in passage geometry, one- way rules, internal entrance capacities, or subpopulation routes [27], [28]. Existing work also rarely separates mechanism di- agnostics from strategy-level management objectives, although evacuation-model verification  studies emphasize component- level testing and emergent-behavior validation [29].

We  propose  a  macroscopic  crowd  modeling  and  control optimization framework for the “Bund sightseeing platform– multi-stepped passage” open scenic area scenario. The frame- work connects multi-stage pedestrian movement, passage di- rection management, geometric guidance, and entrance release control in one simulation-and-optimization workflow. It com- pares  candidate  strategies  under  limited  simulation  budgets, rather  than  relying   only  on  empirical  rules   or  post-event assessment.

This paper makes four contributions.

1)  We  formulate  a  multi-stage,  multi-route  macroscopic model   for   the   open   scenic-area   class   represented by  the  Bund  sightseeing  platform  and  its  connected passages.  Unlike   single-origin–destination   or   single- potential Hughes-type formulations, the model lets entry, sightseeing, exit, return movement, and route redistribu- tion arise through shared congestion.

2)  We  develop  a  Bellman–conservation-law  operator  that combines hard direction rules, anisotropic mobility guid- ance,  and  internal  entrance-rate  metering.  This  uni- fied macroscopic operator represents one-way, two-way, and closed channels, channel pre-alignment, open-but- metered  entrances,  and  upstream  waiting  under  mass- consistent dynamics.

3)  We   establish   an   evaluation   protocol   that   separates behavioral-mechanism  diagnostics  from  strategy-level

management   objectives.    The   protocol    first   checks whether  direction  constraints  and  geometric  guidance behave  as  intended,  and  then  compares  policies  by efficiency, high-density exposure, load balance, entrance waiting, and control smoothness.

4)  We   design  a  hierarchical   constrained  mixed-variable black-box optimization framework (HCMBO) for joint direction and entrance-rate control. The method exploits the direction–capacity  structure and uses unified high- fidelity rechecking to compare candidate policies under the tested budget.

II.  RELATED WORK

A.  Massive Crowd Management and Control

Crowd management  studies in urban centers have mainly used boundary regulation, facility layout, and decision-making models [30], [31]. Zhong et al. [32] designed boundary control strategies for urban traffic-flow networks and proved conver- gence to uncongested states under disturbances through Lya- punov analysis [33]. Wadoo and Kachroo [34] used advection and diffusion control to prevent blocking and shock waves in one-dimensional evacuation. Qin [35] proposed a unit sliding- mode controller based on integral barrier Lyapunov functions for multi-directional disturbance propagation. Zhu et al. [36] combined policy learning and neural networks to adjust inflow rates and free-flow speeds in heterogeneous corridors.

Infrastructure-based regulation has also been  studied. Ob- stacle  placement  can  change  outflow,  velocity,  and  local density  in  merging  areas  [25],  [37].  Yang  et  al.  [38],  [39] used  Model  Predictive  Control  (MPC)  to  optimize  barrier lengths  at  subway  bottlenecks  and  later  included  entrance limiters, gates, and escalator directions. Carmona and Paricio- Garcia  [40] proposed  dynamic exit-choice recommendations using  multinomial  Logit  models,  but  the  approach  depends on  discrete  speed  instructions  (e.g.,  “stop,”  “walk,”  “run”) transmitted through electronic wristbands [41], which are hard to deploy in open urban areas.

Other studies use optimization and reinforcement learning. Liao et al.  [42] formulated fence layout as  subset selection.

Subsequent  work   [28]  used  LSTM  features   and  Proximal Policy  Optimization  (PPO)  [43]  for  real-time  entrance-flow regulation. Differential Evolution (DE) [44] has been used to optimize guardrail layout and flow guidance in scenarios such as Chengdu East Railway Station, with average travel time and crowd pressure as objectives [45].

B.  Macroscopic  Crowd Modeling

Macroscopic crowd models treat pedestrian flows as contin- uous media, offering computational efficiency and analytical tractability compared to microscopic approaches. Henderson [46]  first  applied  fluid  dynamics to pedestrian traffic in the early  1970s, comparing crowd motion to the behavior of gas or fluid particles.

Hughes  [47] pioneered  a  first-order continuum theory for pedestrian  flow,  in  which  walking  speed  depends  on  local density through a fundamental diagram, and movement direc- tion follows the steepest-descent path of a potential function representing  the  remaining  travel  time  to  exits.  The  model links congestion and route choice through this potential. Ling et al.  [48] generalized the Hughes formulation and solved it with a weighted essentially non-oscillatory (WENO) scheme for the conservation law and a fast-sweeping method for the eikonal equation.

Several extensions modify the density, perception, and con- straint  terms  in  macroscopic  models.  Colombo  et  al.   [49] proposed  nonlocal  flux  models  that  replace  pointwise  den- sity  with  a  neighborhood-averaged  quantity  in  the  velocity function  to   account  for   anticipatory  behavior.  Later   work included  anisotropic  perception  fields  and  wall  or  obstacle boundary effects, together with high-resolution schemes and well-posedness results for architectural domains  [50]. Maury et  al.  [51]  introduced  a  projection  model  for  the  hard  in- compressibility constraint, formulating the actual velocity as a  least-squares  projection  of  the  desired  velocity  onto  the set of feasible non-overlapping configurations. The same idea was later recast as a Wasserstein gradient flow with density constraints  [52],  using  the  Jordan–Kinderlehrer–Otto  (JKO) scheme [53] to keep density below its upper bound.

To  overcome  non-physical  behaviors  such  as  supersonic information propagation that may arise in first-order models, Aw and Rascle  [54] introduced a second-order model based on a generalized momentum conservation law that preserves anisotropy. The Aw–Rascle (AR) framework was subsequently adapted to pedestrian flows  [55], where the congestion pres- sure  term  successfully  reproduces  characteristic  phenomena including stop-and-go waves and spontaneous lane formation.

Beyond  the  classical  Hughes-type  framework,  extensions have incorporated control and guidance mechanisms relevant to crowd management. Anisotropic formulations using mobil- ity tensors have been developed to represent channel geome- tries and directional preferences [56], [57], allowing for con- tinuous geometric guidance without explicit prohibition. Con- strained  Hamilton-Jacobi-Bellman  (HJB)  formulations  have been employed to model strict directional constraints such as one-way corridors, with the Bellman optimality principle pro- viding a natural framework for unifying anisotropic geometric guidance with discrete directional constraints.

Despite  these  advances,  existing  approaches  often  treat control  parameters  as  fixed  or  manually  tuned.  The  joint optimization of discrete channel direction configurations and internal entrance capacity profiles remains underexplored, par- ticularly for large-scale urban pedestrian zones with multiple behavioral stages and heterogeneous route choices.

III.  METHODOLOGY

A.  Problem Definition and Overall Framework

We address a multi-channel crowd management problem in open scenic areas. The proposed method combines a Bellman– conservation-law  crowd  dynamics  model   [47],  [48]  with  a joint direction-capacity control formulation. The controllable management variable is defined as

z = (s, q) ∈ Z,                                (1)

where s denotes the discrete direction states of the channels, and q denotes the time-varying internal entrance capacity pro- file. The behavioral preference parameter pˆ and the geometric guidance parameter η0  are treated as fixed model inputs. For a given control variable  z, the lower-level  simulator returns density fields, velocity fields, entrance waiting, and realized channel throughputs. The upper-level optimizer then evaluates efficiency, safety, load balance, waiting, and smoothness met- rics, and searches for a better control policy under a limited simulation budget. The overall method framework is shown in Fig. 2.

We model direction rules and entrance capacities as coupled controls.  The  direction  configuration  s  changes   the  local feasible direction set, whereas the entrance capacity q changes the realized admitted flux at channel entrances. Together, they affect channel loading, high-density exposure, and upstream waiting.  We   therefore  formulate  crowd  management  as  a mixed-variable simulation-based optimization problem, rather than  solving  direction  selection  and  capacity  assignment  as two independent subproblems [28], [58], [59].

B.  Bellman–Conservation-Law Crowd Dynamics

1)  Conservation Law and Speed Function:  We consider a bounded two-dimensional  domain  Ω ⊂ R2  representing the pedestrian  walking  area.  Let  Ωw   ⊂ Ω  denote  the  walkable region, ρ(x, t) the total crowd density, and v(x, t) the local mean velocity. Mass conservation is governed by

The speed magnitude is determined by local congestion. We adopt the Greenshields-type speed function

where  vmax   is  the  free-flow  speed,  ρmax   is  the  maximum admissible density, and  (·)+  denotes nonnegative truncation. This  function  captures  the  feedback  that  pedestrians  move close to free-flow speed in low-density regions and slow down as density approaches congestion.

Direction

configuration s

+1: forward one-way

-1:  reverse one-way 0:  bidirectional

∅:  closed

Entrance-rate

control q

Piecewise admissible rates

q+c (t),qc— (t)

2. Control Variables

Joint control variable z=(s ,q).

geometry

| 1. Scenario and Model Params |  |
| --- | --- |
|  | Walkable domain Ω w and channel Obstacles and internal entrances Behavioral preference ˆp Geometric guidance parameter η 0 |

Weighted objective

J  =  Σ5k=1

|  | 4. Performance Evaluation |  |  |
| --- | --- | --- | --- |
|  |  | Efficiency J 1 travel time Safety J2 density exposure Load balance J3=Var(Rc) Waiting J4 Smoothness J5 |  |

select z∗

| 5. Upper-level HCMBO Optimizer |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Feasible 1 dirseccatnionds |  |  |  |
|  | 2 | Capacity map q = Ts(x) |  |  |
|  | 3 Seaxh[0d,om1]aind |  |  |  |
|  | 4 siMumullit-fidionell. |  |  |  |
|  | 5 | Final re-eval. |  |  |
| Best policy under limited simulation budget |  |  |  |  |

4

|  | 3. Crowd Dynamics Simulator Bellman-Conservation Solver S(z;ˆp,η 0) |  |
| --- | --- | --- |
| A | (a) Multi-stage / multi-route density de- composition ρ = Σ o Σ r ρo,r (b) Total density and speed update Density-dependent speed f(ρ) decreases with ρ (c) Direction constraints and guidance Allowed directions U(x;s) Anisotropic metric M(x;η 0) (d) Constrained Bellman equation Compute potential and optimal direction (e) Finite-volume conservation update Advance ρo,r using upwind fluxes (f) Internal entrance flux control Actual flux capped by q±c (t) Simulation Outputs. Density field ρ(x,t). Potential field ϕ(x,t). Velocity field v(x,t). Entrance wait blocking B. Channel throughput Rc. (g) Stage transition update Route switching governed by ˆp. |

A

feedback update z = (s , q)

Fig. 2.  Method framework for joint direction and entrance-rate optimization. Fixed scenario inputs, behavioral preferences, and geometric guidance parameters define the lower-level Bellman–conservation-law crowd simulator. The controllable direction configuration and entrance-rate profile are jointly evaluated by efficiency, safety, load-balance, waiting, and smoothness objectives, and are updated by the upper-level HCMBO optimizer.

The  classical  Hughes  model  assumes  that  all  movement directions are feasible and isotropic [47]. This assumption is insufficient for crowd management  scenarios involving  one- way rules, temporary closures, entrance metering, and channel- axis alignment. The following subsections introduce admissi- ble  direction  sets,  anisotropic  mobility  tensors,  and  internal entrance capacity control into the Bellman–conservation-law framework.

2)  Multi-Stage Multi-Route Density Decomposition:  Visi- tors in open scenic areas usually experience several behavioral stages, such as entry, touring, exit, and return. To represent this behavior  chain, the total  density is  decomposed into  stage– route subpopulations:

where  σ  denotes  the  behavioral   stage  and  r  denotes  the route  type  within  that  stage.  All  subpopulations  share  the speed function f(ρ) determined by the total density, but they have different target regions, potential functions, and velocity directions.

For subpopulation (σ, r), the density evolves according to

If pedestrians  in  subpopulation  (σ, r)  complete  the  current stage in target region Gσ,r  and switch to next-stage route k with probability pˆ(σ,r)→k , the transition term is

Q(σ,r)→ (σ+1,k)(x, t) = pˆ(σ,r)→k κσ,r χGσ,r (x)ρσ,r (x, t),  (6)

where  κσ,r   is  the  stage-switching  rate,  with  dimension  of inverse  time,  and  χGσ,r     is  the  indicator  function   of  the

completion  region.  The  parameter  pˆ represents  historically inferred  or  scenario-given  route  preferences  and  is  fixed  in the optimization problem [12], [15], [16].

C.   Direction Constraints and Geometric Guidance

# 1)  Direction Configuration Variable: Let the direction state

of channel c be

sc  ∈ {+1, —1, 0, 。},                             (7)

where +1,  —1, 0, and 。 denote reference forward one-way, reference  reverse  one-way,  bidirectional,  and  closed  states, respectively.  If  τc (x)  is  the  reference  tangent  direction  of channel  c,  the  local  admissible  direction  set  in  the  channel region is defined as

Thus,  sc   changes  the  local  feasible  direction  set.  A  closed channel has no admissible direction, a one-way channel retains only one reference tangent, and a bidirectional channel retains both opposite tangential directions.

2) Anisotropic Mobility Tensor: The admissible set U(x;s) represents  hard  feasibility,  but  it  does  not  capture  the  soft geometric guidance induced by narrow channels. Pedestrians approaching a channel often pre-align with the channel axis before entering it. To represent this effect, an anisotropic mo- bility tensor M(x;η0 ) is introduced [50], [56], [57]. Equiva- lently, it is the inverse of a local travel-cost metric. For channel

c, let τc  and nc  denote its tangent and normal directions. The mobility tensor is defined as

Mc (x;η0 ) = βc  (η0 τc τTc + nc nTc) ,        η0  ≥ 1,     (9)

where  βc  is  a  channel  weight  and  η0  controls the  mobility preference strength along the channel axis. Larger η0  increases tangential mobility, equivalently reducing the effective cost of tangential motion and promoting upstream pre-alignment. The parameter η0 is fixed and is not optimized as a control variable.

3)  Constrained  Bellman   Equation:   Given   U(x;s)   and M(x;η0 ), the potential function  ϕ(x, t) represents the min- imum remaining cost to the corresponding target region. The discrete Bellman update is

The  excess  demand   (A±c(t) — ˆA±c(t))+   remains  upstream

and is recorded as waiting or queueing mass. This treatment changes channel usage and risk distribution while preserving the mass balance of the conservation-law update.

E.  Numerical Solver and Simulation Operator

The  walkable  region,  obstacles,  channel  areas,  and  inter- nal  entrance  interfaces  are  discretized  on  a  uniform  two- dimensional grid. At each time step, the total density is first computed  from  all  stage–route  subpopulations  and  used  to update  f(ρ).  For  each  subpopulation,  the  admissible  direc- tion  set  and  mobility  tensor  are  then  constructed,  and  the corresponding discrete Bellman equation is solved. After the

=  u iroi[r]in,als

For any feasible control variable z = (s, q), the complete

The optimal direction is                                                                  applied to update the subpopulation densities.

u* (x, t) = arg min(···),                    (11)

u∈U(x;s)

and the velocity is

v(x, t) = f(ρ(x, t))u* (x, t).                (12)

For  the  multi-stage  multi-route  model,  each  subpopulation (σ, r) uses its own target-specific potential ϕσ,r  while sharing the  same  total-density  congestion  feedback.  Thus,  direction rules, geometric guidance, and density-dependent congestion are coupled in a single Bellman–conservation-law model.

D.  Internal Entrance Capacity Control

Direction  rules   alone  cannot  represent  the   management action  in  which  a  channel  is  open  but  its  entrance  must be metered  [58],  [59],  [60]. For  each channel, we introduce maximum internal entrance rates

q = {q+c(t), qc—

where q+c(t) and qc— (t) denote the maximum admitted rates

at the reference forward and reverse entrances of channel c. These controls act on internal channel entrance fluxes rather than  on  external  boundary  inflows.  In  physical  units,  both

the attempted flux A±c(t) and the capacity bound q±c(t) are

entrance flow rates measured in ped/s.

The direction state and entrance capacity must satisfy the hard consistency constraints

sc  = +1 ⇒ qc— (t) = 0,

(14)

sc  = —1 ⇒ q+c(t) = 0,

sc  = 。 ⇒ q+c(t) = qc— (t) = 0, sc  = 0 ⇒ q+c(t) + qc— (t) ≤ q-c.

Let  A±c(t) be  the  attempted  entrance  flux  induced  by  free

movement. The actually admitted flux is

ˆ±A c(t) = min{A±c(t), q±c(t)}.                (15)

When  A±c(t)  >  0,  the  flux  across  the  internal  entrance  is

scaled by

numerical workflow defines a lower-level simulation operator

(ρ,ϕ, v, B, R) = S(z;pˆ,η0 ),                   (17)

where  B  records  entrance  waiting  or  blocking  and  R   = (R1,..., RC ) records realized cumulative throughputs of the channels. The channel balance metric below is computed from the realized throughputs Rc , not from the prescribed capacity

limits q±c (t).

F.   Evaluation Metrics and Optimization Objective

1)  Efficiency,  Safety,  and  Load  Balance:   Following  re- cent passenger-flow-control and crowd-evacuation simulation- optimization  studies  [37],  [40],  [60],  the  total  travel  time metric is                              T

J1 (z) =     ρ(x, t) dx dt.                 (18)

The high-density exposure metric is

J2 (z) =   1ρ(x,t)>ρsafe  dx dt.             (19)

The safety threshold is the density above which exposure is counted, and is fixed at ρsafe  = 3.5 ped/m2  in both the four- channel and Bund scenarios. Let

be  the  realized   cumulative  throughput  of  channel   c.  The channel load imbalance is

J3 (z) = Var(R1 (T),..., RC (T)).             (21)

# 2)  Entrance  Waiting  and  Control  Smoothness:   Entrance waiting is penalized as

For piecewise-constant capacity profiles, smoothness is mea- sured by

Here  k  indexes  the  capacity-control  time  segment.  J4   dis- courages  excessive  entrance  metering  that  creates  upstream queues, while J5  discourages  abrupt high-frequency changes in capacity profiles [41], [58], [59], [60].

3)  Normalization and Scalar Objectives:  The raw metrics above  have   different  physical  units   and  numerical   scales. Therefore, the reported  scalar objectives are computed from dimensionless normalized quantities. Let

Mref  = M0 + Min ,        Te  = T,

where  M0  is the initial crowd mass,  Min  is the cumulative external inflow during [0, T], and Te is the evaluation horizon. Let |Ωw | be the walkable area, Rtot  = Σ Rc (T), and

The basic normalized terms are

For segmented entrance-rate profiles, the smoothness term is normalized by the corresponding entrance-rate bounds:

Here Nq  is the number of active finite differences, q-σc is the

reference upper bound of the corresponding entrance, and εR and εq  only avoid zero denominators. Thus,  J-1  is a relative system-residence measure. The term  J-2  is  the  average hard exposure or soft safety-risk intensity over the walkable space- time domain  [7]. The term J-3  is  zero for perfectly balanced realized  channel  use  and  approaches  one  when  all  realized channel throughput concentrates on one channel.

The reported terms use scale factors

.                                  (24)

The main experiments use

(a1 , a2 , a3 , a4 , a5 ) = (1, 10 −3 , 1, 1, 1).

terms and should not be read directly as space-time percent- ages;  the  basic  normalized  value  is    Given weights λ = (λ1,...,λ5 ), the scalarized objective is

with the main-experiment setting

(λ1 ,λ2 ,λ3 ,λ4 ,λ5 ) = (1, 1, 1, 1, 0.1).

and the optimization problem is

,        z = (s, q).                     (26)

TABLE I

RAW UNITS OF THE OBJECTIVE TERMS AND THEIR NORMALIZERS

Raw unit

Reported

Term

Normalizer (same unit)

| J1 | ped·s | MrefTe (ped·s) | dimensionless |
| --- | --- | --- | --- |
| J2 | m2 ·s | ¦Ωw¦Te (m2·s) | dimensionless |
| J3 | ped2 | DR (ped2) | dimensionless |
| J4 | ped·s | MrefTe (ped·s) | dimensionless |
| J5 | (ped/s)2 | (q-σc)2 (ped/s)2 | dimensionless |

The  paper  solves  this  fixed-weight  scalarized  management problem.  Different  weights  can  represent  different  manage- ment  preferences,  but  the  algorithm  itself  does  not  depend on one  specific objective~ term being independently optimal. Although  J,  J(3) ,  and  Jk   are  dimensionless  ranking  quan- tities,  their  interpretation  is  anchored  by  the  normalization above [45], [61]. When physical interpretation is needed, the experiments also report raw or ratio-based diagnostics such as channel-flow shares, system mass, and density peaks. Table I lists the physical units of the raw objective terms  and their normalizers; in each case the term~ and its normalizer  share the same unit, so every reported Jk  is dimensionless.

For mechanism-verification experiments and fixed-strategy diagnostic experiments that only test direction rules, geometric guidance, or fixed entrance-rate responses, we use the reduced three-term objective

J(3) (z) = λ 1 1 (z) + λ2 2 (z) + λ3 3 (z).         (27)

In these experiments, the entrance-waiting term  4  is either reported  only  as  a  diagno~stic  quantity  or  is  not  applicable, and the smoothness term J5  is meaningful only for segmented capacity-profile optimization. Equivalently, reduced-objective experiments use λ4  = λ5  = 0. The numerical values of J and J(3)  are not compared across experiment groups.

G.  Hierarchical Constrained Mixed-Variable Black-Box Opti- mization

Since s is discrete, q is continuous or piecewise continuous, and each objective evaluation requires the expensive simula- tor S, the resulting problem is  an expensive mixed-variable black-box optimization problem [62], [63], [64]. We propose HCMBO,  a Hierarchical  Constrained Mixed-variable Black- box  Optimization  method,  to  exploit  the  structural  relation between direction variables and capacity variables.

1)  Direction Candidates and Capacity Parameterization:

HCMBO first generates  a direction candidate  set Scand  sat- isfying  operational  and  connectivity  constraints. For  a  fixed direction configuration s, the entrance capacity profile is rep- resented by a low-dimensional continuous vector x ∈ [0, 1]d and mapped to the actual capacity control by

q = Ts (x),         x ∈ [0, 1]d.                   (28)

The  mapping  Ts   automatically  applies  direction  masks,  ca- pacity  upper  bounds,  closure  constraints,  and  smoothness restrictions.  For  example,  if  a  channel  is  set  to  reference forward one-way, the reverse entrance capacity is set to zero; if a channel is closed, both directional capacities  are  set to zero. Thus, the optimizer searches in the continuous x-space

while all candidates are mapped to capacity profiles consistent with the discrete direction configuration.

2)  Structured  Capacity Search:  For  each  direction  candi- date s, HCMBO performs structured search in the correspond- ing capacity parameter space. The search uses previously eval- uated samples to construct surrogate information for objective values and feasibility, and preferentially selects capacity candi- dates that satisfy constraints and may improve the incumbent objective. This hierarchical design  avoids blind  sampling in the full mixed-variable space and separates discrete direction structure from continuous capacity structure.

Multi-fidelity evaluation can be used during search. Low- or medium-fidelity simulations help identify clearly poor capacity profiles,  whereas  high-fidelity  simulations  are  reserved  for final candidate rechecking and ranking. To  avoid discarding good long-horizon  strategies due to  short-horizon bias, low- fidelity results  are used  only  as  auxiliary  evaluation  signals and do not directly determine the final optimum.

3)  High-Fidelity  Rechecking  and Final  Selection:  All  fi- nalist candidates are re-evaluated under a unified high-fidelity setting. Let CHF  be the candidate set selected for high-fidelity rechecking. The final output is

z* = arg min JHF (z).                        (29)

z∈CHF

This mechanism prevents surrogate error, low-fidelity bias, or intermediate search ranking from directly becoming the final performance claim.

Algorithm 1 gives the reproducible HCMBO procedure used in the experiments. The loop terminates after the finite opti- mization budget has been allocated across direction candidates, and  the  reported  optimum  is  selected  only  after  the  high- fidelity rechecking stage.

Compared    with    generic    mixed-variable    optimization, HCMBO uses the dependence between direction choices and feasible  capacity  profiles.  Once  s  is  fixed,  the  mapping  Ts removes  capacity  variables  that   conflict  with  the   selected direction rules and bounds the remaining entrance-rate profiles. This reduces invalid candidates and allocates the  simulation budget to policies that satisfy the operational constraints.

IV.  NUMERICAL EXPERIMENTS

We validate the proposed model and optimizer on a four- channel  scenic-platform  scenario  abstracted  from  the  Bund sightseeing platform setting. The scenario consists of an inflow region, a Bund sightseeing platform and exit region, a central wall,  and  four  connecting  channels:  upper,  middle,  lower- middle, and bottom channels. Throughout this paper, channel denotes  a model flow entity indexed by  c, whereas passage refers to a real-world walkway at the Bund site that a channel abstracts. The experiments are organized as mechanism val- idation, directional-response analysis, entrance-rate response, horizontal optimizer comparison, and HCMBO ablation.

A.  Experimental Setup

The  domain is  discretized  on  a  128 × 96  grid.  To  report results  in  physical  units,  simulation  quantities  are  mapped

| Algorithm Control | 1 HCMBO for Joint Direction and Entrance-Rate |
| --- | --- |
| Require: Direction constraints Ωs ; simulator S ; objective J; gate-rate bounds q-; budget B ; initial design size n0 ; pool size P; LCB coefficient β; high-fidelity shortlist size K. Ensure: Best high-fidelity policy z ∗ . 1: Generate Scand by enumerating s ∈ Ωs and retaining only configurations satisfying open-channel and connectivity constraints. 2: Rank Scand using the prescribed priority templates and direction proxy score, and then truncate it to the direction-candidate limit. 3: Set bs = ⌊B/¦Scand ¦⌋ and assign one extra evaluation to the first B mod ¦Scand ¦ directions. 4: Initialize the global evaluation log D ← ∅ . 5: for all s ∈ Scand do 6: Construct Ts : [0, 1]ds → q by applying the direction mask, gate bounds q-, and time segmentation. 7: Initialize Ds ← ∅ and generate X0s from high-, medium-, and low-rate profiles plus random fill-up points. 8: for all x ∈ X0s do 9: if ¦Ds ¦ < bs then 10: Evaluate z = (s, Ts (x)) with the optimization-fidelity simulator and append (x, z, J(z)) and feasibility metrics to Ds and D. 11: end if 12: end for 13: while ¦Ds ¦ < bs do 14: Let xbsest be the evaluated point with the lowest objective in Ds. 15: Draw Ps from P uniform samples in [0, 1]ds and max(4, ⌊P/4⌋) Gaussian perturbations of xbsest , clipped to [0, 1]ds . 16: Set the kernel length scale ℓs = max{0.18, 1/√ds }. 17: for all u ∈ Ps do 18: Estimate µˆs (u) by weighted averaging with wi (u) = exp(-∥xi - u∥22/(2ℓ2s)) over (xi , zi , Ji ) ∈ Ds. 19: Set σˆs (u) = min{1, min(xi,zi, Ji)∈Ds ∥u - xi ∥2 /√ds }. 20: Compute as (u) = µˆs (u) - β std(Ds )σˆs (u). 21: end for 22: Select xnext ← arg minu∈Ps as (u). 23: Evaluate znext = (s, Ts (xnext )) and append the result to Ds and D. 24: end while 25: end for 26: Select the top K unique controls CHF from D by the optimization-fidelity objective. 27: for all z ∈ CHF do 28: Re-evaluate z under the unified high-fidelity simulator setting to obtain JHF (z). 29: end for 30: return z ∗ = arg minz∈CHF JHF (z). |  |

through a representative calibration based on three reference scales: a grid cell  size of ∆x = 1.0 m,  so that the domain spans  128 m × 96 m; a free-flow walking speed of 1.5 m/s; and  a jam density of 5.0 ped/m2. The induced time  step is then ∆t = 0.2 s, and pedestrian counts follow by integrating density over physical area, so that a fully occupied cell holds ρmax ∆x2   =  5 ped.  As  a  coherence  check,  a  fully  packed domain of this size would hold at most ρmax  × 128 × 96 ≈ 6.1 × 104  ped,  which bounds all reported pedestrian counts. These  physical  units  are  adopted  only  for  readability  and reflect a representative assumption; they do not represent any

# Fig. 3.  Four-channel scenic-platform simulation scenario used in the numer- ical experiments.

specific venue.

The  channel  width  is  4  m,   and  the  controlled  internal entrance  interface  width  is   16  m.  The  free-flow   speed  is vmax  = 1.5 m/s, the maximum density is ρmax  = 5.0 ped/m2 , the initial density is ρinit  = 2.2 ped/m2 ,  and the CFL factor is 0.9.

Mechanism and response experiments use the corresponding validation  horizons.  The   optimizer  comparison  uses  high- fidelity rechecks with 1600 simulation steps, a horizon of 320 s (i.e. ∆t = 0.2 s), and Bellman updates every five steps. Final optimizer rankings are based on the high-fidelity objective in Section  III-F.  Unless  otherwise  stated,  optimization  experi- ments are ranked by the full five-term scalar objective J after high-fidelity rechecking.

Mechanism-validation,  directional-response,  fixed-rate  re- sponse,  and  supplementary  transfer  dia~gnostics~ use  the  re- duced  objective   J J2  + λ3 J3.  For  these reduced-objective experiments, J4  is only~ an entrance-waiting diagnostic  or  is  not  applicable,  and   J5   is  not  applicable. Equivalently, λ4  = λ5  = 0. Therefore, the numerical values of  J  and  J(3)   should  not  be  compared  across  experiment groups.  Comparisons  are  meaningful  only  within  the  same objective definition, normalization rule, and weight setting. A high-fidelity best candidate is called feasible if the density-cap removal mass is no more than 2% of the reference mass.

Unless  otherwise  stated,  cumulative  flow,  waiting  mass, unreleased attempted flow, and  system mass are reported in pedestrians  (ped),  obtained  by  integrating  density  over  the physical cell area. Mass-time quantities are reported in ped·s. Counterflow area-time is reported in m2·s. Channel-flow shares are computed from realized cumulative channel throughputs, Rc (T)/ Σj Rj (T). Scalar objectives and normalized terms are dimensionless. Raw cumulative quantities are rounded to three significant digits in the text, while tables retain four decimals for dimensionless objective terms.

B.  Mechanism  Validation

We first verify whether the two local modeling components have the intended behavioral meanings. The fixed anisotropic mobility tensor M(x;η0 ) should reshape the channel attraction domain and induce entrance pre-alignment, whereas the admis- sible direction set U(x;s) should remove prohibited directions and reroute flow through feasible channels. When M is applied to the middle channel, its flow share increases from 48.78% in the baseline to  100.00%.  When  a one-way rule excludes the originally dominant middle-channel direction, the middle- channel share becomes zero and the flow is redistributed to the upper, lower-middle, and bottom channels with shares 19.25%, 68.76%, and  12.00%, respectively. For the bidirectional val- idation case, we use two diagnostic quantities. The reverse- direction mass time is

RMT ∆x2 ∆t,

which measures the cumulative reverse-direction mass-time (in ped·s) moving against the prescribed channel direction. The counterflow area-time is

which  measures  the  cumulative  area-time  (in  m2·s)  during which opposite-direction subpopulations coexist in the same channel region. In the baseline bidirectional inflow test, the westbound middle-channel share, RMT, and CAT are 73.18%, 3.33 × 103  ped·s,  and  2.43 × 103   m2·s,  respectively.  After imposing the eastbound one-way rule, all three quantities are reduced to zero.

Fig.  4  compares  guidance  applied  to  different  channels. In  each  case,  the  dominant  flow  channel  and  local  density hotspot move  with the  guided  channel. This  supports using the  Bellman–conservation-law   simulator  as  the  lower-level evaluator for subsequent strategy optimization.

C.  Directional-Control Response

We next examine whether the discrete direction configura- tion s creates nontrivial system-level trade-offs. The response experiment  uses  thirteen  interpretable  direction  settings:  an all-free  baseline,  four  single-entry  cases,  four  single-return cases, and four one-channel-closure cases. The direction code is ordered as [T, M, LM, B], where E, W, F, and X denote eastbound entry, westbound return, bidirectional free channel, and closure.

Fig. 5 and Table II show that no single direction configura- tion dominates all objectives. This directional-response exper- iment summarizes the three reported te~rms by~J(3) ; since no entrance-capacity curve is optimized, J4  and  J5  are not used for ranking. C8 (SR-LM, EEWE) obtains the lowest reduced objective  J(3)    =  0.5102,  which  is  only  0.84%  lower  than th~e all-free baseline C1. C1 gives the lowest safe~ty exposure J2  = 0.00785, C3 gives the best efficiency t~erm  J1  = 0.3141, and C9 gives the best load-balance term  J3  = 0.1138. This non-monotonic response makes manual rule selection from a small set insufficient and motivates automatic mixed-variable optimization.

# Fig. 4.  Spatial transferability of the combined U +M mechanism. Moving the controlled channel induces corresponding migration of dominant flow channels, streamlines, and density hotspots.

Fig. 5.  Directional-control design matrix and objective-space trade-off. Color encodes  J(3), and diamonds indicate non-dominated cases.

TABLE II

NON-DOMINATED DIRECTIONAL-CONTROL CASES

| Case | Policy | Code | ~ J1 | ~ J2 | ~ J3 | J(3) | Main characteristic |
| --- | --- | --- | --- | --- | --- | --- | --- |
| C1 | BSL | FFFF | 0.3535 | 0.00785 | 0.1531 | 0.5145 | Lowest exposure |
| C3 | SE-M | WEWW | 0.3141 | 0.01680 | 0.2741 | 0.6049 | Best efficiency but risk- |
| C4 | SE-LM | WWEW | 0.3335 | 0.01662 | 0.3131 | 0.6632 | concentrated Efficient but load-imbalanced |
| C6 | SR-T | WEEE | 0.4029 | 0.01174 | 0.1335 | 0.5482 | High return throughput |
| C8 | SR-LM | EEWE | 0.3395 | 0.01311 | 0.1576 | 0.5102 | Lowest reduced |
| C9 | SR-B | EEEW | 0.4377 | 0.01344 | 0.1138 | 0.5649 | objective Most balanced channel loads |

All objective terms in this table are dimensionless scaled normalized quan- tities.  The  reduced  objective  uses  λ4    =  λ5    =  0  and  is  used  only  for directional-response diagnosis; it is not directly comparable with the full five- term objective in Table IV.

D.  Entrance-Rate Response

Direction  rules  alone  cannot  represent  the  case  in  which a  channel  remains  open  but  its  internal  entrance  must  be

metered.  We  therefore  fix   [T, M, LM, B]  =   [F, E, W, F] and  scan  four  internal  entrance-rate  levels:  no  limit,  high, medium,  and  low.  The  rate  upper  bound  acts  on  internal entrance  crossing  flow.  When   attempted  flow  exceeds  the upper bound, only the admitted flow crosses the interface, and the  unreleased  attempted  flow  remains  upstream  as  waiting mass. The entrance binding-time ratio is defined as the fraction of time  steps in which attempted entrance flow exceeds the rate upper bound and creates unreleased attempted flow. The unreleased attempted flow is

in pedestrian-equivalent demand (ped-eq.); because held-back mass  re-attempts  the  interface  at  later  steps,  Urel   is  a  cu- mulative  time  integral  of  repeatedly  denied  demand  rather than  an  instantaneous   queue  length,   and  may   exceed  the number  of  distinct  waiting  pedestrians.  Its  demand  ratio  is rrel  = Urel dt. This scan is a fixed- rate diagnostic experiment rather than a final optimizer-ranking experiment;  the  table  therefore  reports  J(3) ,  treats  entrance

# TABLE III

RESPONSE UNDER INTERNAL ENTRANCE-RATE UPPER-BOUND LEVELS

| Level | ~ J1 | ~ J2 | ~ J3 | J(3) | Urel | rrel | Binding ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| No limit | 0.6274 | 0.174 | 0.8838 | 1.6851 | 0.00 | 0 | 0.000 |
| High | 0.6262 | 0.358 | 0.8814 | 1.8652 | 965.60 | 70.8% | 0.605 |
| Medium | 0.6228 | 0.443 | 0.8620 | 1.9276 | 1317.04 | 82.6% | 0.666 |
| Low | 0.6181 | 0.466 | 0.8493 | 1.9337 | 1711.56 | 92.1% | 0.721 |

Urel is reported in pedestrian-equivalent demand (ped-eq.), a cumulative time integral of denied entrance demand and not an instantaneous queue length. The safety column is a scaled normalized safety term, not a direct space-time percentage.

Fig.  6.    Objective-term  response  under  internal  entrance-rate  upper-bound levels.  Tightening  the upper bound  changes  safety  exposure,  load balance, and unreleased attempted flow; the latter is reported in pedestrian-equivalent demand (ped-eq.).

waiting  as  a  diagnostic  quantity,  and  sets  the  smoothness contribution to zero because each policy uses a fixed rate level.	Table  III  and  Fig.  6  show  that  tightening  the  rate  upper bound does not simply improve safety. From the no-limit case to  the  low  level,  unreleased  attempted  flow  increases  from 0 to  1.71 × 103  ped-eq., corresponding to rrel   = 92.1% of attempted entrance demand. The binding ratio incr~eases from

0 to 0.721, and the scaled normalized safety term J2  increases from 0.174 to 0.466. Thus, q is a genuine continuous control variable that changes system response and must be optimized jointly with s.

E.  Horizontal Optimization Comparison

The horizontal comparison evaluates Prior Baseline, Ran- dom  Search,  Pure  Simulated  Annealing,  Enumeration  plus Differential  Evolution,  TPE-Mixed  BO,  and  HCMBO.  All optimizers  use  B  =  400  candidate  evaluations  and  top-10 high-fidelity  rechecks   over  seeds  11,  23,  37,  51,  and  73. The final HCMBO data  are taken from the G7-D mainline, whereas the other baselines are taken from the G6 horizontal comparison under the same high-fidelity protocol.

Table  IV  reports  the  smallest  mean,  median,  and  best objective for HCMBO, together with the highest feasible rate among the tested methods. Its mean objective is 3.9% lower than TPE-Mixed BO, since (2.7403-2.6338)/2.7403 = 3.9%. Because this is a normalized scalar objective, the percentage reduction reflects the fixed-weight management  score, not a direct reduction in travel time or density exposure alone. In paired comparisons, HCMBO has a lower objective than TPE- Mixed  BO  in  four  of five  seeds,  with  an  average  objective

TABLE IV

HIGH-FIDELITY PERFORMANCE UNDER THE FULL FIVE-TERM

DIMENSIONLESS OBJECTIVE

| Method | Mean objective | Std. | Median | Best Worst | Feasible rate |
| --- | --- | --- | --- | --- | --- |
| HCMBO | 2.6338 | 0.3106 | 2.5065 | 2.4495 3.1851 | 100% |
| TPE-Mixed BO | 2.7403 | 0.2475 | 2.6288 | 2.5180 3.1345 | 80% |
| Pure SA | 2.9248 | 0.1386 | 2.9203 | 2.7468 3.1049 | 60% |
| Random Search | 3.0112 | 0.1319 | 2.9797 | 2.8821 3.2287 | 40% |
| Enum-DE | 3.0597 | 0.1932 | 3.1004 | 2.7329 3.2500 | 40% |
| Prior Baseline | 5.4774 | 0.0000 | 5.4774 | 5.4774 5.4774 | 0% |

The  reported  objective  is  the  full  dimensionless  scalar  objective   J   =

Σ5k=1    after  unified  high-fidelity  rechecking.  These  values  are  not

directly comparable with the reduced diagnostic objective J(3)  in Tables II, III, and VII.

## Fig.  7.    Best  high-fidelity  full  objective  J  across  seeds.  The  objective  is dimensionless and lower is better. Range bars show the seed-wise spread, and the short vertical mark indicates the median.

## Fig. 8.  Best-so-far convergence curves under the same 400-evaluation budget. The inset highlights the final-stage separation among methods.

difference of -0.1066. It is also lower than Random Search in  all  five  seeds  and  lower than Pure  SA  and Enum-DE in four  of  five  seeds.  The  remaining  high  HCMBO  objective is  mainly  caused by  a  larger  safety-exposure  term,  so  later versions  should  add  more  explicit  safety  constraints  during search.

Figs.  7–9  compare  the  seed-wise  objectives,  convergence traces,  and  selected  entrance-rate  profiles.  TPE-Mixed  BO decreases quickly in the early evaluations, whereas HCMBO

Fig.  9.    Best-solution  entrance-rate  profiles  for  Random  Search,  Pure  SA, Enum-DE, TPE-Mixed BO, and HCMBO. The common direction structure is [T, M, LM, B] = [W, E, E, W] for several methods, but the segment-wise rate allocations differ substantially.

continues to reduce the best-so-far objective during the middle and late stages. The control profiles indicate that methods with the same direction code can still choose different segment-wise rate allocations, leading to different final objectives.

F.  HCMBO Ablation

The  ablation  experiments  test  the  contribution  of  each control and search component. Optimizing only directions with a fixed high rate gives a best high-fidelity objective of 3.3751, and optimizing only rates under prior directions gives 3.3140; both  are  higher  than  the  joint   optimization  result.   Short- horizon low-fidelity hard  screening raises the best  objective to  2.9441 and  selects  an  infeasible  best  candidate,  because early  simulations  do  not  reliably  capture  long-horizon  en- trance waiting and load redistribution. Removing the entrance- waiting penalty slightly lowers the scalar objective to 2.6015, but  increases  unreleased  attempted  flow  to  approximately 1.30 × 104  ped-eq. The waiting term should therefore remain in the management objective.

Table  V  shows  that  the  final  HCMBO  mainline  has  the lowest  scalar  objective  among  the  tested  variants.  Queue- aware LCB and RF-style constrained BO reduce unreleased attempted flow to approximately 9.64 × 103  ped-eq., but their scalar objective increases to 2.6162. Thus, queue information is useful for executable and safety-aware extensions, whereas the final mainline should retain structured direction-wise rate search as the primary budget allocation mechanism.

# TABLE V

SEARCH-MECHANISM ABLATION UNDER SEED 23

| Variant | Best Feasible objective rate | ~ J2 | Unreleased flow (ped-eq.) | Main meaning |
| --- | --- | --- | --- | --- |
| HCMBO | 2.4495 100% | 1.6759 | 1.02×104 Structured direction-wise rate search |  |
| Queue-aware LCB | 2.6162 100% | 1.8521 | 9.64×103 Queue-aware acquisition |  |
| RF-style constrained BO | 2.6162 100% | 1.8521 | 9.64×103 Constraint surrogate variant |  |
| Adaptive racing | 2.6561 100% | 1.9474 | 1.28×104 Stage-wise budget concentration |  |
| Adaptive racing | 2.6683 100% | 1.7862 | 1.06×104 Combined racing |  |
| + queue-aware Trust-region search | 3.0317 100% | 1.9224 | and queue signal 1.59×104 Local trust-region search |  |

Unreleased flow is the cumulative unreleased attempted entrance flow over the  full  horizon,  in  pedestrian-equivalent  demand   (ped-eq.);  it  integrates repeatedly denied demand and is not an instantaneous queue length.

G.  Transfer Validation on a Bund-Inspired Scenario

The preceding experiments use a standardized four-channel platform  scenario to  verify modeling mechanisms  and  opti- mization behavior under controlled conditions. As a supple- mentary validation, we further construct a simplified abstract scenario inspired by the  spatial form of the  Shanghai Bund waterfront. This experiment does not claim to reproduce all architectural details of the real site. Instead, it preserves the Bund sightseeing platform, the lateral passage connections, the major inflow location, and the dominant movement direction, so as to examine whether an HCMBO-derived policy can still change the load distribution in a more realistic spatial outline.	The computational grid has resolution 512 × 224. Because this scenario preserves the real Bund outline, its physical scale is calibrated from georeferenced control points that match the scene coordinates to  satellite latitude  and longitude;  a four- point affine fit gives 0.435 m per scene unit with sub-1.5 m residual, which corresponds to a physical cell  size of about 1.75 m.  The  512 × 224 grid  therefore  spans  approximately

894  m × 391  m,  the  induced  time  step  is  ∆t   ≈  0.44  s, and  pedestrian  counts  again  follow  by  integrating  density over physical area (a cell area of about 3.0 m2 ). Pedestrians enter from the  outer-side entrance, pass through the middle connecting channels, and move toward the Bund sightseeing platform and its central target region. The inflow-rate multi- plier is 3.0, corresponding to an actual inflow rate of about 3.35 ped/s;  the  maximum  density  is  ρmax    =  4.0  ped/m2 , and the stage-switching rate is κσ,r  = 32 in simulation time units. The experimental channels channel_1, channel_2, channel_3,  and  channel_4  correspond  to  the  locally named Bund Passages 3, 4, 5, and 6, respectively. Both the controlled  and  uncontrolled  groups  use   1600  steps  over  a physical horizon of about 698 s. For the controlled group, the transferred entrance-rate bounds are multiplied by a capacity- scale factor of 3, providing a less restrictive entrance-capacity setting.

The controlled group uses the best HCMBO policy obtained

Fig.  10.   Real Bund scene and its simplified simulation abstraction. The abstraction retains the Bund sightseeing platform boundary, the main entrance, and the passage connections used for control validation. The satellite basemap was obtained from Google Maps. The right-side local photographs were shot in the Shanghai Bund waterfront passage area by the Tongji Public Safety Experimental Team, June 21, 2026.

TABLE VI

EFFECTIVE ENTRANCE-RATE UPPER BOUNDS IN THE BUND TRANSFER

EXPERIMENT

| Experimental channel | Local Entrance side passage name | Segment 1 (ped/s) | Segment 2 (ped/s) |
| --- | --- | --- | --- |
| channel_1 | Bund Passage Forward 3 entrance | 3.45 | 8.38 |
| channel_1 | Bund Passage Reverse 3 entrance | 3.17 | 0.00 |
| channel_2 | Bund Passage Forward 4 entrance | 8.38 | 2.36 |
| channel_2 | Bund Passage Reverse 4 entrance | 0.00 | 0.00 |
| channel_3 | Bund Passage Forward 5 entrance | 0.00 | 0.00 |
| channel_3 | Bund Passage Reverse 5 entrance | 3.57 | 6.95 |
| channel_4 | Bund Passage Forward 6 entrance | 5.28 | 3.03 |
| channel_4 | Bund Passage Reverse 6 entrance | 3.09 | 2.96 |

from  the  multi-direction  high-fidelity  recheck.  Its  reduced high-fidelity objective in the optimization stage is 0.7350, and its  direction  structure  leaves  the  upper  and  lower  channels bidirectional  while  assigning  opposite  dominant  directions to  the  two  middle  channels.  Table  VI  reports  the  effective two-segment  entrance-rate  upper  bounds  after  applying  the capacity-scale factor of 3. The rate upper bounds are measured in ped/s.

Table VII compares the controlled and uncontrolled groups

under the same high-fidelity protocol. It separates dimension- less normalized terms from raw  simulation diagnostics. The reduced objective J(3)  decreases from  1.0140 to 0.7410, cor- responding to an absolute reduction of approximately 0.2730 and a relative reduction of 26.9~%. The largest reduction comes from the  load-balance term  J3 ,  which~decreases  by  51.9%, from ~0.6043 to  0.2908.  In  contrast,   J1   increases  by  8.7% and J2  increases by 25.7%. This indicates that the transferred policy changes how pedestrians use the channels, rather than merely reducing all density values uniformly.

The channel-flow shares in Table VIII further explain this reduction. In the uncontrolled group, 82.30% of the channel flow concentrates on channel_4, i.e., Bund Passage 6. After applying the HCMBO policy, the flow is redistributed mainly to  channel_2  and  channel_3,  whose  combined  share reaches approximately 91.34%.

Fig.  11  visualizes  the  density  evolution  at  approximately 22,  153,  393,  and  654 s.  Both  groups  initially  form  a local density concentration near the right-side target region of the Bund  sightseeing  platform,  reflecting  the  same  inflow  and target-attraction  structure.  At  later  stages,  the  uncontrolled case remains more concentrated around the terminal channel and the adjacent Bund sightseeing platform area, whereas the controlled case develops a more stable flow band through the middle-channel group. This visual evidence is consistent with the channel-share statistics and confirms that the main effect of the HCMBO policy is load redistribution across channels.

## Fig. 11.  Density evolution comparison between uncontrolled and controlled cases at approximately 22, 153, 393, and 654 s. The controlled policy redistributes the later-stage flow from the terminal channel toward the middle-channel group.

TABLE VII

CONTROLLED AND UNCONTROLLED RESULTS IN THE BUND TRANSFER

EXPERIMENT

| Dimensionless term | Controlled Uncontrolled Relative change |  |  |  |
| --- | --- | --- | --- | --- |
| J(3) ~ J~1 J~2 J3 | 0.7410 1.0140 -26.9% 0.4145 0.3813 +8.7% 0.0357 0.0284 +25.7% 0.2908 0.6043 -51.9% |  |  |  |
| Raw diagnostic | Controlled Uncontrolled Unit |  |  |  |
| Final cumulative sink flow Final cumulative inflow Final system mass Peak density maximum Domain-mean density (incl. empty space) Attempted entrance flow Actual entrance flow Unreleased attempted flow |  | 1.33×103 2.32×103 1.53×103 ≈ 4.0 3.47×10-3 1.49×103 1.13×103 364 | 1.67×103 2.34×103 1.20×103 ≈ 4.0 3.21×10-3 – – – | ped ped ped ped/m2 ped/m2 ped-eq. ped ped-eq. |

Sink flow, inflow, system mass, and actual entrance flow are pedestrian counts (ped).  Attempted  and  unreleased  entrance  flow  are  cumulative  pedestrian- equivalent demand (ped-eq.): they integrate repeatedly denied demand over the horizon and may exceed the number of distinct waiting pedestrians. Entries marked  -  are  not  applicable  because  the  uncontrolled  case  activates  no internal metering interface. The  domain-mean  density is  averaged  over the full walkable domain, including empty space, and is therefore much smaller than the local peak.

TABLE VIII

CHANNEL-FLOW SHARES IN THE BUND TRANSFER EXPERIMENT

| Channel | Local name | Controlled | Uncontrolled |
| --- | --- | --- | --- |
| channel_1 channel_2 channel_3 channel_4 | Bund Passage 3 | 2.83% | 0.32% |
|  | Bund Passage 4 | 30.38% | 1.35% |
|  | Bund Passage 5 | 60.96% | 16.03% |
|  | Bund Passage 6 | 5.83% | 82.30% |

Channel-flow shares are computed as Rc(T)/ Σ the realized cumulative admitted throughput of channel c during the simulation horizon.

The transfer experiment exposes a trade-off. Compared with the  uncontrolled  ~group,  the  controlled  group  has  a  higher efficiency term  (J1   increases  from  0~.3813 to  0.4145)  and  a slightly higher safety-exposure term (J2  increases from 0.0284 to 0.0357). Its cumulative sink flow is also smaller (1.33×103 versus  1.67×103   ped),  and  its  final  system  mass  is  larger (1.53×103  versus  1.20×103   ped).  The  transferred HCMBO policy  therefore  should  not  be  read  as  a  uniform  safety  or efficiency improvement. Under the present reduced-objective weights, it lowers J(3)  mainly by redistributing channel load,

while giving up part of exit throughput and travel efficiency. The capacity-scale factor of 3 reduces unreleased attempted flow to 364 ped-eq., but it does not remove the efficiency cost. A deployment-oriented version should add explicit throughput constraints and waiting upper bounds, and should test several objective-weight settings before selecting a policy.

## V.  CONCLUSION

This  paper  proposed  a  macroscopic  crowd-management framework for open pedestrian zones with passage-direction rules,  geometric  guidance,  multi-stage  route  behavior,  and internal entrance-rate control. The framework couples a crowd- flow  simulator  with  HCMBO  so  that  direction  settings  and entrance-rate profiles can be optimized under a limited simu- lation budget.

The  experiments  indicate  that  the  two  local  modeling components  behave  as  intended  and  that  direction  control and  entrance-rate  control  are  not  interchangeable.  Direction settings  create  non-monotonic  trade-offs   among  efficiency, safety exposure, and load balance. Entrance-rate limits change unreleased attempted flow, binding-time ratios, and local risk patterns. Across five random seeds with unified high-fidelity rechecking, HCMBO gives the lowest mean full management objective  among  the  tested  baselines,   with  a  3.9  percent lower  fixed-weight  score  than  TPE-Mixed  BO  and  a   100 percent feasible rate. The ablation study attributes this result to joint direction-rate optimization, avoiding hard short-horizon screening, and structured direction-wise rate search.

The  Bund-inspired  transfer  experiment  indicates  that  an HCMBO-derived policy can alter flow organization in a sim- plified  real-world  spatial  abstraction.  The  reduced  objective decreases by 26.9 percent because the policy redistributes flow away from the dominant terminal channel and improves load balance  across  the  middle-channel  group.  The  same  policy also  produces  higher  residual  mass,  lower  exit  throughput, and  unreleased  attempted  entrance  flow.  The  transfer  result supports mechanism transfer and load redistribution, but it is not a deployable plan for the Bund area.

The  study  has  several  limitations.  The  experiments  use simplified  geometries,  fixed  objective  weights,   and  finite random  seeds.  One  HCMBO  seed  still  has  relatively  high safety  exposure.  The  model  also  does  not  yet  include  real crowd calibration, online demand updates, or executable field constraints.  Future  work  should  address  these  limits  before operational deployment, especially by adding stronger safety

and throughput constraints and by testing multiple demand and weight settings.

ACKNOWLEDGMENT

This research was supported by the National Natural Sci- ence  Foundation  of  China   (No.  72374154)  and  the  20th Experimental Teaching Reform Special Foundation of Tongji University (No. 0800104347). The authors deeply appreciate this support.

REFERENCES

[1]  P.   Charalambous,   J.   Pettre,   V.   Vassiliades,   Y.   Chrysanthou,   and N. Pelechano, “Greil-crowds: Crowd simulation with deep reinforcement learning and examples,” ACM Trans. Graph., vol. 42, no. 4, p. Article 137, 2023.

[2]  H.  Jiang,  X.  Zhang,  Y.  Dong,  and  J.  Wang,  “Data-driven  predictive modeling of citywide crowd flow for urban safety management: A case study of beijing, china,” Journal of Forecasting, vol. 44, no. 2, pp. 730– 752, 2025.

[3]  D. Shi, J. Li, Q. Wang, J. Chen, R. Lovreglio, J. T. Y. Lo, and J. Ma, “Modeling and simulation of crowd dynamics at evacuation bottlenecks during flood disasters,”  Transportation Research Part E: Logistics and Transportation Review, vol. 198, p. 104064, 2025.

[4]  R. Zhao, D. Wang, Y. Wang, C. Han, P. Jia, C. Li, and Y. Ma, “Macro- scopic  view:  Crowd  evacuation  dynamics  at  t-shaped  street junctions using a modified aw-rascle traffic flow model,” IEEE Transactions on Intelligent Transportation Systems, vol. 22, no. 10, pp. 6612–6621, 2021.

[5]  B. L. Zhou, X. O. Tang, and X. G. Wang, “Learning collective crowd behaviors  with  dynamic  pedestrian-agents,”  International  Journal  of Computer Vision, vol. 111, no. 1, pp. 50–68, 2015.

[6]  T. Bosse, M. Hoogendoorn, M. C. A. Klein, J. Treur, C. N. van der Wal, and A. van Wissen, “Modelling collective decision making in groups and crowds: Integrating  social contagion  and interacting emotions, beliefs and intentions,” Autonomous Agents and Multi-Agent Systems, vol. 27, no. 1, pp. 52–84, 2013.

[7]  H. Liang, L. Yang, J. Du, C.-W. Shu, and S. C. Wong, “Modelling crowd pressure  and  turbulence  through  a  mixed-type  continuum  approach,” Transportmetrica B:  Transport Dynamics, vol.  12, no.  1,  p.  2328774, 2024.

[8]  Z. Shahhoseini and M. Sarvi, “Pedestrian crowd flows in shared spaces: Investigating the impact of geometry based on micro and macro scale measures,”  Transportation Research Part B: Methodological, vol.  122, pp. 57–87, 2019.

[9]  C.-Z. T. Xie, J. Xu, B. Zhu, T.-Q. Tang, S. Lo, B. Zhang, and Y. Tian, “Advancing crowd forecasting with graphs across microscopic trajectory to  macroscopic  dynamics,”  Information  Fusion,  vol.  106,  p.  102275, 2024.

[10]  N. W. F. Bode, M. Chraibi, and S. Holl, “The emergence of macroscopic interactions  between  intersecting  pedestrian  streams,”  Transportation Research Part B: Methodological, vol. 119, pp. 197–210, 2019.

[11]  F. Bourdin, “Splitting scheme for a macroscopic crowd motion model with congestion for a two-typed population,” Networks and Heteroge- neous Media, vol. 17, no. 5, pp. 783–801, 2022.

[12]  L. Y. Wang and W. X. Huang, “Visitors’ consistent stay behavior pat- terns within free-roaming  scenic architectural complexes: Considering impacts  of temporal,  spatial,  and  environmental  factors,” Frontiers  of Architectural Research, vol. 13, no. 5, pp. 990–1008, 2024.

[13]  X. Yang, R. Zhang, F. Pan, Y. Yang, Y. Li, and X. Yang, “Stochastic user equilibrium path planning for crowd evacuation at subway station based on social force model,”  Physica A: Statistical Mechanics and its Applications, vol. 594, p. 127033, 2022.

[14]  J.  Zhong,  T.  Cheng, W.  L.  Liu,  P.  Yang,  Y.  Lin,  and  J.  Zhang,  “An evolutionary  guardrail  layout  design  framework  for  crowd  control  in subway stations,”  IEEE Transactions on Computational Social Systems, vol. 10, no. 1, pp. 297–310, 2023.

[15]  P. X. Liu, X. Xiao, J. Zhang, R. H. Wu, and H. L. Zhang, “Spatial config- uration and online attention: A space syntax perspective,” Sustainability, vol. 10, no. 1, 2018.

[16]  A. M.  Caldeira  and E.  Kastenholz,  “Spatiotemporal tourist behaviour in urban destinations: a framework of analysis,” Tourism Geographies, vol. 22, no. 1, pp. 22–50, 2020.

[17]  Jingmin Zhitongche  Shanghai,  “Tourism,  shopping festival,  and  com- mercial  markets  bring  overlapping  visitor  flows:   Shanghai  huangpu police implement holiday security measures,” Tencent News.  [Online]. Available: https://news.qq.com/rain/a/20240503A05LRO00, May 2024, (in Chinese). Accessed: Jun. 25, 2026.

[18]  L. Yin, “Instantaneous visitor flow in the core bund scenic area reaches 71,000; police activate crowd-guidance measures,” China News Service, Chinanews.com. [Online]. Available: https://www.chinanews.com.cn/tp/ hd2011/2019/10-03/906114.shtml,  Oct.  2019,  (in  Chinese).  Accessed: Jun. 25, 2026.

[19]  M. Haghani, M. Coughlan, B. Crabb, A. Dierickx, C. Feliciani, R. van Gelder,  P.  Geoerg,  N.  Hocaoglu,  S.  Laws,  R.  Lovreglio,  Z.  Miles, A.  Nicolas,  W.  J.  O’Toole,  S.  Schaap,  T.  Semmens,  Z.  Shahhoseini, R. Spaaij, A. Tatrai, J. Webster, and A. Wilson, “A roadmap for the future of  crowd  safety  research  and  practice:  Introducing  the  swiss  cheese model of crowd safety and the imperative of a vision zero target,” Safety Science, vol. 168, p. 106292, 2023.

[20]  J. Zhong, D. Li, W. Cai, W.-N. Chen, and Y.  Shi,  “Automatic crowd navigation path planning in public scenes through multiobjective differ- ential evolution,”  IEEE Transactions on  Computational Social Systems, vol. 11, no. 1, pp. 905–918, 2024.

[21]  H. Gu, S. Kim, and M. Jung, “Data-driven urban planning for proactive crowd  management:  Lessons  from  the  2022  seoul  halloween  crowd crush,” Cities, vol. 168, pp. 1–14, 2026.

[22]  K. Rezaee, M. R. Khosravi, H. Attar, V. G. Menon, M. A. Khan, H. Issa, and L. Qi, “Iomt-assisted medical vehicle routing based on uav-borne human crowd sensing and deep learning in smart cities,” IEEE Internet of Things Journal, vol. 10, no. 21, pp. 18 529–18 536, 2023.

[23]  Shanghai Municipal Public Security Bureau Huangpu Branch, “Notice on the issuance  of the  “Implementation Plan for  Safety Management of Key Areas such as the Bund and Nanjing Road for the 2023 New Year”,” 2022, (in Chinese).

[24]  CCTV,  “Beijing  dongcheng  police  take  multiple  measures  to  handle large crowds at nanluoguxiang,” Oct. 2021, (in Chinese).

[25]  H. Yu, N. Jiang, M. Li, X. Jia, J. Shi, E. Wai Ming Lee, and L. Yang, “Regulating  multi-directional  passenger  flow:  Impact  of  obstacle  po- sition  and  flow level  on pedestrian merging process,”  Tunnelling and Underground Space Technology, vol. 157, p. 106336, 2025.

[26]  D. Wu and H. Diao, “Optimization of security police deployment and scheduling for large-scale events,”  Xitong Gongcheng Lilun yu Shijian (Syst. Eng. — Theory & Pract.), vol. 42, no. 03, pp. 789–800, 2022, (in Chinese).

[27]  N.  Molyneaux  and  M.  Bierlaire,  “Controlling  pedestrian  flows  with moving walkways,”   Transportation Research Part  C: Emerging  Tech- nologies, vol. 141, p. 103672, 2022.

[28]  X.  C.  Liao,  W.  N.  Chen,  X.  Q.  Guo,  J.  Zhong,  and  D.  J.  Wang, “Drift: A dynamic crowd inflow control system using lstm-based deep reinforcement  learning,”   IEEE  Transactions  on  Systems,  Man,   and Cybernetics: Systems, vol. 55, no. 6, pp. 4202–4215, 2025.

[29]  E.  Ronchi,  D.  Nilsson,  E.  D.  Kuligowski,  R.  D.  Peacock,  and  P.  A. Reneke,  “Assessing  the  verification  and  validation  of  building  fire evacuation models,” Fire Technology, vol. 52, no. 1, pp.  197–219, 2016.

[30]  H. M. Al-Ahmadi, I. Reza, A. Jamal, W. S. Alhalabi, and K. J. Assi, “Preparedness for mass gatherings: A simulation-based framework for flow control and management using crowd monitoring data,” Arabian Journal for Science  and Engineering, vol. 46, no.  5, pp. 4985–4997, 2021.

[31]  L. Feng and E. Miller-Hooks, “A network optimization-based approach for  crowd  management  in  large  public  gatherings,”  Transportation Research Part C: Emerging Technologies, vol. 42, pp. 182–199, 2014.

[32]  R. X. Zhong,  Y. P.  Huang,  C.  Chen, W.  H.  K. Lam, D. B. Xu,  and A.  Sumalee,  “Boundary  conditions  and  behavior  of  the  macroscopic fundamental diagram based network traffic dynamics: A control systems perspective,”  Transportation Research Part B: Methodological, vol. 111, pp. 327–355, 2018.

[33]  R. Zhao, Q. Liu, Q. Hu, D. Dong, C. Li, and Y. Ma, “Lyapunov-based crowd stability analysis for asymmetric pedestrian merging layout at t- shaped street junction,”  IEEE Transactions on Intelligent Transportation Systems, vol. 22, no. 11, pp. 6833–6842, 2021.

[34]  S. A. Wadoo and P. Kachroo, “Feedback control of crowd evacuation in  one  dimension,”   IEEE  Transactions  on  Intelligent  Transportation Systems, vol. 11, no. 1, pp. 182–193, 2010.

[35]  W. Qin, “Unit sliding mode control for disturbed crowd dynamics system based on integral barrier lyapunov function,” IEEE Access, vol. 8, pp. 91 257–91 264, 2020.

[36]  Y. Zhu, D. Zhao, and H. He, “Optimal feedback control of pedestrian flow  in  heterogeneous  corridors,”   IEEE  Transactions  on  Automation Science and Engineering, vol. 18, no. 3, pp. 1097–1108, 2021.

[37]  Y. Zhou, M. Zhong, Z. Li, X. Xu, F. Hua,  and R. Pan,  “Simulation- based  adaptive  optimization  for  passenger  flow  control  measures  at metro  stations,”  Simulation Modelling Practice and  Theory, vol.  138, p. 103021, 2025.

[38]  X.  Yang,  B.  Shi,  G.  Zhang,  and  Y.  Li,  “Optimization  modeling  of automatic crowd regulation at bottlenecks of subway system: A model predictive  control  approach,”  Physics Letters A,  vol.  531, p.  130180, 2025.

[39]  X. Yang, G. Zhang, B. Shi, C.-Z. Xie, and B. Zhang, “Partition inde- pendent  control  and  collaborative  optimization  of high-density  crowd in subway stations,”  Chaos,  Solitons  & Fractals, vol. 200, p.  117108, 2025.

[40]  M. A. Lopez-Carmona and A. Paricio-Garcia, “Crowd evacuation man- agement  with  optimal  time-multiplexed  exit-choice  recommendations through  simulation  optimization,”  Advanced  Theory  and  Simulations, vol. 7, no. 7, p. 2400265, 2024.

[41]  M.  A.  Lopez-Carmona  and  A.  Paricio  Garcia,  “Linear  and  nonlinear model  predictive  control  (mpc)  for  regulating  pedestrian  flows  with discrete  speed  instructions,”  Physica A:  Statistical Mechanics  and  its Applications, vol. 625, p. 128996, 2023.

[42]  X.  C.  Liao,  W.  N.   Chen,  X.   Q.  Guo,  J.  Zhong,   and  X.  M.   Hu, “Crowd management through optimal layout of fences: An ant colony approach based on crowd simulation,” IEEE Transactions on Intelligent Transportation Systems, vol. 24, no. 9, pp. 9137–9149, 2023.

[43]  J. Schulman, F. Wolski, P. Dhariwal, A. Radford, and O. Klimov, “Prox- imal policy optimization algorithms,” arXiv preprint arXiv:1707.06347, 2017.

[44]  K.  Price,  R.  M.  Storn,  and  J.  A.  Lampinen,  Differential  Evolution: A  Practical Approach  to  Global  Optimization.      Springer  Publishing Company, Incorporated, 2014.

[45]  W. L. Liu, Y. Chen, X. Li, J. Zhong, R. Chen, and H. Jin, “Automatic optimization  of  guidance  guardrail  layout  based   on  multi-objective evolutionary  algorithm,”   Complex  System  Modeling  and  Simulation, vol. 4, no. 4, pp. 353–367, 2024.

[46]  L. F. Henderson, “The statistics of crowd fluids,” Nature, vol. 229, no. 5284, p. 381–383, 1971.

[47]  R.  L.  Hughes,  “A  continuum  theory  for  the  flow  of  pedestrians,” Transportation Research  Part B: Methodological,  vol.  36,  no.  6,  pp. 507–535, 2002.

[48]  L.  Huang,  S.  C.  Wong,  M.  Zhang,  C.-W.  Shu,  and  W.  H.  K.  Lam, “Revisiting hughes’ dynamic continuum model for pedestrian flow and the  development  of  an  efficient   solution  algorithm,”   Transportation Research Part B: Methodological, vol. 43, no. 1, p. 127–141, 2009.

[49]  R.   M.    COLOMBO,    M.   GARAVELLO,   and   M.    L´ECUREUX-

MERCIER, “A class of nonlocal models for pedestrian traffic,” Math- ematical Models and Methods in Applied Sciences, vol. 22, no. 04, p. 1150023, 2012.

[50]  B.  Raimund,  G.  Paola,  I.  Daniel,  and  V.  Luis  Miguel,  “A  non-local pedestrian flow model accounting for anisotropic interactions and do- main boundaries,”  Mathematical Biosciences and Engineering, vol.  17, no. 5, p. 5883–5906, 2020.

[51]  B. Maury and J. Venel, “A mathematical framework for a crowd motion model,” Comptes Rendus Mathematique, vol. 346, no. 23, p. 1245–1250, 2008.

[52]  B. Maury, A. Roudneff-Chupin, F. Santambrogio, and J. Venel, “Han- dling congestion in crowd motion modeling,” Networks and Heteroge- neous Media, vol. 6, no. 3, p. 485–519, 2011.

[53]  R. Jordan, D. Kinderlehrer, and F. Otto, “The variational formulation of the fokker–planck equation,”  SIAM Journal on Mathematical Analysis, vol. 29, no. 1, p. 1–17, 1998.

[54]  A. Aw and M. Rascle, “Resurrection of ”second order” models of traffic flow,” SIAM Journal on Applied Mathematics, vol. 60, no. 3, p. 916–938, 2000.

[55]  J.-P.  Lebacque,  S.  Mammar,  and  H.  Haj-Salem,  “The  aw–rascle  and zhang’s  model:  Vacuum  problems,  existence  and  regularity  of  the solutions  of  the  riemann  problem,”  Transportation  Research  Part  B: Methodological, vol. 41, no. 7, p. 710–721, 2007.

[56]  E. Cristiani, F. S. Priuli, and A. Tosin, “Modeling rationality to control self-organization of crowds: An environmental approach,” SIAM Journal on Applied Mathematics, vol. 75, no. 2, pp. 605–629, 2015.

[57]  F. Priuli, A. Tosin, and E. Cristiani, “On the control of self-organized systems: The case of pedestrian flows,” in Proceedings of the 5th Inter- national Conference on Pedestrian and Evacuation Dynamics, 2015.

[58]  Z. Jiang, W. Fan, W. Liu, B. Zhu, and J. Gu, “Reinforcement learning approach for coordinated passenger inflow control of urban rail transit in

peak hours,”   Transportation Research Part C: Emerging Technologies, vol. 88, pp. 1–16, 2018.

[59]  J.  Liu,  L.  Hu,  X.  Xu,  and  J.  Wu,  “A  queuing  network  simulation optimization method for coordination control of passenger flow in urban rail transit stations,” Neural Computing and Applications, vol. 33, no. 17, pp. 10 935–10 959, 2021.

[60]  H. Hu, R. Zhang, Y. Hao, Y. He, and Z. Liu, “Optimization model for passenger flow control and service capacity allocation in subway station pedestrian facility networks,” Sustainability, vol.  17,  no.  21, p.  9816, 2025.

[61]  R. T. Marler and J. S. Arora, “Survey of multi-objective optimization methods  for  engineering,”  Structural  and Multidisciplinary  Optimiza- tion, vol. 26, no. 6, pp. 369–395, 2004.

[62]  H.  M.  Sheikh  and  P.  S.  Marcus,  “Bayesian  optimization  for  mixed- variable,  multi-objective  problems,”   Structural  and  Multidisciplinary Optimization, vol. 65, no. 11, p. 331, 2022.

[63]  J. C. Ramirez, R. Le Riche, O. Roustant, G. Perrin, C. Durantin, and A. Gliere, “A comparison of mixed-variables bayesian optimization ap- proaches,”  Advanced Modeling and Simulation in Engineering Sciences, vol. 9, no. 1, p. 6, 2022.

[64]  K.  Dreczkowski,  A.  Grosnit,  and  H.  Bou-Ammar,  “Framework  and benchmarks  for  combinatorial  and  mixed-variable  bayesian  optimiza- tion,” in  Advances in Neural Information Processing Systems 36, 2023.

Bingyu Wei received the B.E. degree from Tongji Univer- sity, Shanghai, China, in 2023, where he is currently pur- suing the Ph.D. degree in control science and engineering. His research interests include pedestrian behavior, crowd stability, and computer vision.

Cuiling Li received the B.E. degree in automation from Northwestern  Polytechnical  University,  China,  the  M.E. degree in automation from Lanzhou University of Tech- nology, China, and the Ph.D. degree in systems engineer- ing  from  Tongji  University,  China,  in  1998,  2002,  and 2007, respectively. Her research interests include pedes- trian dynamics, crowd stability, and artificial intelligence.

Rongyong Zhao received the B.E. degree in mechanical			science  from  Jinan  University,  China,  the  M.E.  degree	in  mechanical  automation  from  Lanzhou  University  of	Technology, China, and the Ph.D. degree in control theory		and engineering from Tongji University, China, in  1999,		2002,  and  2005,  respectively.  He  is  currently  an  Asso-	ciate Professor with Tongji University, Shanghai, China.	His research interests include pedestrian dynamics, crowd evacuation, and computer vision.

Zhengyang   Shi   is   currently   a   senior  engineer  with Huangpu  Public  Security  Bureau,  Shanghai,  China.  His research  interests  include  large-scale  pedestrian-crowd safety, large language models, and artificial intelligence. He  has  published  more  than  ten  SCI  papers  in  related fields and holds several authorized patents.

Yuxin Cai received the B.E. degree from China University of Petroleum  (Beijing),  China, in  2024. He is currently pursuing  the  master’s  degree  in  digital  information  at Tongji University, Shanghai, China. His research interests include pedestrian anomaly detection, public  safety, and computer vision.

Lingchen  Han  received  the  B.E.  degree  from  Shang- hai  Polytechnic  University,  Shanghai,  China,  in  2024. He  is  currently  pursuing  the  master’s  degree  in  digital information  at  Tongji  University,  Shanghai,  China.  His research  interests  include  pedestrian  anomaly  detection, crowd stability, and computer vision.